ATNP/SGB1/WP111

Proposed Draft Guidance Material on the 3rd Edition Enhancements ICS3-08

(Suppression of re-advertisement of routes in the case of changing mobile subnetwork connectivity)

Prepared by Stéphane Tamalet

(France)

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

SG-B1/1

Honolulu, Hawaii

27-28 Feb 2001

SUMMARY

This Working Paper provides draft Guidance Material for the draft 3rd edition enhancements ICS3-08 (Suppression of re-advertisement of routes in the case of changing mobile subnetwork connectivity)

TABLE OF CONTENTS

1	Introduction	i

2	Proposed Changes to the CAMAL	ii

# Introduction

This document provides draft Guidance Material for the draft 3rd edition enhancements ICS3-08 (Revision of TP4 Acknowledgement Timer Value). It has been prepared for input to WG-B.

# Proposed Changes to the CAMAL

3.4.10 Route Initiation

3.4.10.1 Purpose of Route Initiation

3.4.10.2 Ground-Ground Route Initiation

3.4.10.3 Air-Ground Route Initiation

3.4.10.3.2 Communications Environment

3.4.10.3.3 Summary of Procedures

3.4.10.3.4 Initial Route Initiation

3.4.10.3.5 Route Initiation in CLNP

3.4.10.3.5.1 As a result of the handling of the Join Event or the “polling” procedure described above, a virtual circuit will have been established between Airborne and Ground Routers. The Mobile SNDCF specified in the ATN ICS SARPs should also have been assigned to support the use of this virtual circuit by CLNP. As with ground-ground Route Initiation, it is now necessary for the IS-SME to add to each Router s FIB, a route to the NET of the remote Router s Network Entity, using the newly established virtual Circuit.

3.4.10.3.5.2 However, all each Router knows at this point is the DTE Address of the other Router. In order to avoid the maintenance problem inherent in managing lookup tables that would enable a correspondence to be made between a DTE Address and a NET, a dynamic procedure has been specified by the ATN ICS SARPs.

3.4.10.3.5.3 An ISO/IEC 9542 IS Hello (ISH) PDU is used for this purpose. This is sent either as data, once the connection has been established, or as part of the Call Request/Call Confirm dialogue when “Fast Select” is supported by the air-ground subnetwork. Both Airborne and Ground Routers generate an ISH PDU that reports their NET to the other Router. On receipt of an ISH PDU, each Router updates its FIB with a route to the remote Router, using the NET supplied by the ISH PDU and associating this NET with the subnetwork connection over which the ISH was received, as the forwarding path.

3.4.10.3.5.4 The initial ISH PDU exchange is also used to inform the peer ATN Router about those baseline and enhanced capabilities which are supported by the sending ATN Router over the air/ground adjacency and about the characteristics of the air/ground subnetwork used to exchange the ISH PDU.  More specifically, the initial ISH PDU exchange is used as a support for the exchange of the following information:

- The Airborne routeur uses its ISH PDU to indicate support of the use or of the non-use of IDRP. The support of the use or of the non-use  of IDRP is indicated by the value of the Network Selector (NSEL) field of the Network Entity Title (NET) of the Airborne routeur. The NSEL field of the NET of the Airborne routeur is set to zero if the Airborne routeur supports the use of IDRP over the air-ground link. It is set to the hexadecimal value 'FE' if the Airborne BIS does no support IDRP, and intends to use the procedure for the non-use of IDRP (see 3.4.10.4).

- The Air/Ground routeur uses its ISH PDU to inform the Airborne routeur about the traffic type restriction(s) and the supported ATSC Class(es) of the air-ground subnetwork over which the ISH PDU is exchanged. This information is conveyed in the option part of the ISH PDU, and included in an ATN specific optional parameter: the Mobile Subnetwork Capabilities Parameter. The ATN Airborne router uses this information to make appropriate routing decisions when downlinking CLNP packets.

- The Air/Ground and the Airborne routeurs compliant with the most recent edition of the ATN Technical Provisions use another ATN specific optional parameter of the ISH (the ATN Data Link Capabilities Parameter) to signal their enhanced capabilities to each other over mobile data links. This parameter allows for the negotiation of new technical features during routing initiation, and allows for the inter-working of ATN Systems of different vintages by restricting enhanced systems to the invocation and use of only those enhanced capabilities for which the peer system indicates support. The ATN Data link Capabilities parameter has the form of a bits map where the setting of a bit indicates support of the corresponding capabilities.

An ATN Air/Ground routeur may use the ATN Data Link Capabilities parameter to indicate:

- Whether it supports the generation of IDRP UPDATE BISPDUs without A/G Subnetwork Type Security Tag (in order to efficiently use scarce air/ground resources),

- Whether it intends to perform IDRP Type 2 authentication in subsequent BISPDU exchanges with the peer Airborne Router; and

- Whether it requires the receiving Airborne Router to downlink its Public-Key Certificate for subsequent validation

An ATN airborne routeur may use the ATN Data Link Capabilities parameter to indicate:

- its capability of receiving and processing IDRP UPDATE BISPDUs which do not contain A/G Subnetwork Type Security Tag, and

- Whether it intends to perform IDRP Type 2 authentication in subsequent BISPDU exchanges with the peer Air/Ground Router.

3.4.10.3.6 Route Initiation in IDRP

3.4.10.3.6.1 Route Initiation in IDRP in the air-ground case is then almost identical to the ground-ground case, except that the ATN ICS SARPs require that one Router is in the passive mode and the other in the active mode. This is because the efficiency improvement gained by this approach is worthwhile in the air-ground environment, and the active and passive roles can be unambiguously identified when ICAO air-ground data subnetworks are used.

3.4.10.3.6.2 The ATN ICS SARPs specify that for air-initiated air-ground subnetworks (i.e. AMSS and VDL), that the Ground Router takes on the active role and the Airborne Router takes on the passive role. For ground-initiated air-ground subnetworks (i.e. Mode S), the ATN ICS SARPs specify that the Airborne Router takes on the active role and that the Ground Router takes on the passive role. This approach will permit the exchange of route initiation data to take place in the shortest timeframe.

3.4.10.3.6.3 The Adjacent BIS MO, if it does not already exist, must be created in response to a notification that an ISH PDU has been received over a new subnetwork connection. It is necessary to create this MO in response to receipt of the ISH PDU, because it is not realistic to pre-configure an Adjacent BIS MO for every Airborne or Ground Router to which it could be connected.

3.4.10.3.6.4 An IDRP “Start Event” is then invoked by the IS-SME, provided that a BIS-BIS connection does not already exists with the remote system. If a BIS-BIS connection does already exist then, as in the ground-ground case, and for the same reasons, the IS-SME must cause the IDRP Decision Process to be re-run.

3.4.10.3.7 Route Advertisement

3.4.10.3.7.1 Introduction

3.4.7.10.3.7.1.1 Once the IDRP connection is established, the airborne routeur and the air-ground routeur exchange IDRP routing information. The routing information is conveyed within IDRP Update BISPDUs. Each IDRP Update BISPDU encodes one or more routes, and comprises a set of destinations, together with information describing the path to such destinations.

3.4.7.10.3.7.1.2 Information about the traffic that a route can carry and the Air/Ground Subnetworks that the route may pass over is conveyed within the Security Path Attribute of the IDRP Update BISPDUs. The setting, update and processing of the Security Path Attribute is governed by a set of ATN specific rules that are specified in section 5.8.3.2 of the ATN Technical Provisions. The rules to be applied by an airborne router differ from those to be applied by an air/ground router.

3.4.7.10.3.7.1.2 This section provides guidance material on the way airborne routers and air/ground routers update and process the security path attribute of routes advertised over mobile subnetworks.

3.4.10.3.7.2 Routes exchanged in the air-to-ground (downlink) direction

3.4.10.3.7.2.1 Setting of the Security Path Attribute for an Airborne routeur

3.4.10.3.7.2.1.1 An airborne routeur is expected to provide each adjacent air/ground BIS with a route to NSAPs and NETs contained within the Mobile Routing Domain, for the Security RIB-Att. A route "for the security RIB-Att" must comprise the ATN Security Path Attribute.

3.4.10.3.7.2.1.2 When advertising such a route, an airborne routeur is not required to fill the Security Path Attribute with any Security Tag Sets. The Security Path attribute must be present, but it may comprise no Air/Ground Subnetwork Type Security Tag Set and no ATSC Class Security Tag Set. This is indeed the Air/Ground routeur, which on receipt of the route is tasked to update the Security Path attribute of the route with the appropriate Security Tag Sets.

Note.- An airborne routeur is allowed to fill the Security Path Attribute of the route with Air/Ground Subnetwork Type Security Tag Sets and/or with an ATSC Class Security Tag Set on the basis of locally known, pre-stored, security information. This is notably done by some router implementations that have been designed in such a way that they can be profiled to act as an Air/Ground routeur or as an Airborne routeur. However, this information will be ignored and overwritten by the air/ground routeurs. The only consequence of such a behaviour is therefore a waste of the bandwidth.

3.4.10.3.7.2.1.2 An Airborne routeur is not expected to re-advertise its route(s) every time a change in subnetwork connectivity occurs over the Adjacency with an Air/Ground Routeur. When a new subnetwork connection via a new mobile subnetwork becomes available, or when connectivity via a mobile subnetwork disappears, this is indeed the Air/Ground Routeur that is tasked to update the Security Path Attribute of the route to airborne Routing Domain before re-advertising this route to other ground routeurs.

3.4.10.3.7.2.2 Update of the Security Path Attribute by an Air-Ground routeur

On receipt of an IDRP Update BISPDUs from an airborne routeur, an air/ground routeur is required to update the security information of the route, before storing this route in its IDRP Loc-RIB. As a result of the update, the Security Path Attribute of the route is expected to comprise the following information:

- For each Air/Ground Subnetwork supporting the adjacency between the Air/Ground routeur and the Airborne BIS, there must be one A/G Subnetwork Type Security Tag Set encoding the mobile subnetwork type (Mode S, VDL, AMSS, Gatelink or HF) and the restrictions on the traffic types that are not allowed to pass over  the subnetwork.

- If one or more Air/Ground Subnetworks supporting the adjacency between the Air/Ground routeur and the Airborne BIS are approved for ATSC use, there must be an ATSC Class Security Tag Set identifying the ATSC class(es) supported by the adjacency. The ATSC Class Security Tag Set name must be set depending upon whether the mobile adjacency is supporting ATSC traffic only or both ATSC and non-ATSC traffic: if all subnetwork connections supporting the adjacency are restricted to the support of ATSC traffic only, then the Tag Set Name is set to the hexadecimal value '07'. If at least one subnetwork connection supporting the adjacency is available to non-ATSC traffic, then the Tag Set Name is set to the hexadecimal value '06'.

Note.- An update of the security information of the route may also occur when the Air/Ground BIS copies a route received from an Airborne Router from the Loc-RIB to an Adj-RIB-Out before advertising the route to an adjacent Ground Router. In this case, the security information of the route is updated further on the basis of the security characteristics of the ground adjacency that exists between the air/ground and the ground routeur. This update is governed by the rules for the update of the security path attribute of routes exchanged over ground-ground adjacencies. These rules are not documented in this section.

The Air/Ground BIS is tasked to re-update the Security Path attribute of the route every time a change in the mobile subnetwork connectivity occurs over the adjacency with the Airborne routeur. For example, if a new subnetwork connection via a new mobile subnetwork becomes available to support the adjacency, then the air-ground BIS is expected to add a new Air/Ground Subnetwork Type Security Tag Set, and update the ATSC Class Security Tag Set of the route(s) already received from the airborne router. Likewise, if a Leave event occurs for one of the subnetwork connectivity supporting the adjacency with the airborne routeur, then the air-ground BIS is expected to remove the corresponding Air/Ground Subnetwork Type Security Tag Set, and update the ATSC Class Security Tag Set of the route(s) already received from the airborne router.

When the Security Path Attribute of the route to the airborne Routing Domain changes in the Loc-RIB, the Air/Ground BIS also updates that routing information in its Adj-RIB-Outs taking into account the possible additional security restrictions of the ground-ground adjacencies. The route is then re-advertised to the adjacent ground routeurs. A change in the mobile subnetwork connectivity over the adjacency with an airborne routeur, implies therefore a re-advertisement of the associated route by the Air-Ground routeurs to its adjacent ground routeurs (provided that the local policy permits such advertisement).

3.4.10.3.7.3 Routes exchanged in the ground-to-air (uplink) direction

The way airborne and air/ground routeurs update and process the security path attributes of routes exchanged in the uplink direction over mobile subnetworks depends on whether the Air-Ground and the Airborne router support the enhanced capability to process Update BISPDUs without A/G Subnetwork Type Security Tag.

The support of this enhanced capability is indicated by the value of the bit 0 of the Data Link Capabilities Parameter in the ISH PDU generated by an airborne or air/ground routeur routeur. An air/ground routeur sets this bit if it supports the capability to generate, over the air/ground link, Update BISPDUs without A/G Subnetwork Type Security Tags. An airborne BIS sets this bit if it is able to receive and process Update BISPDUs without A/G Subnetwork Type Security Tags. If the Data Link Capabilities Parameter is not present in the ISH PDU, then it must be assumed that that enhanced capability is not supported.

When that enhanced capability is not supported by one of the peer routeur over a mobile adjacency, the airborne and the air/ground routeur are expected to process uplink IDRP routes according to the procedures that are hereafter referred as the "baseline procedures for the exchange of routing information in the uplink direction".

On the other hand, if the air/ground and the airborne routeurs have both indicated support of that enhanced capability, they are expected to process uplink IDRP routes according to the procedures that are hereafter referred as the "enhanced procedures for the exchange of routing information in the uplink direction".

3.4.10.3.7.3 Baseline procedures for the exchange of routing information in the uplink direction

Note: As explained in the previous section, the "baseline" procedures'for the exchange of routing information in the uplink direction only applies over mobile adjacencies when one or both of the peer routeurs do not support the enhanced capability to process Update BISPDU without Air/Ground Subnetwork Security Tag.

3.4.10.3.7.3.1 Update of the Security Path Attribute by an Air-Ground routeur

An Air/Ground routeur is expected to provide each adjacent airborne routeur with a route to NSAPs and NETs contained within its own local Routing Domain, for the Security RIB-Att. It is also recommended to provide the adjacent airborne routeurs with an aggregated route to all NSAPs and NETs within the local ATN Island, and an aggregated route to all ground ATN NSAPs and NETs, for the Security RIB-Att. A route "for the Security RIB-Att" must comprise the ATN Security Path attribute.

Notes: Other routes may also be provided to the adjacent airborne routeurs. This is dependent on the local routing policy.

The routes to be advertised to adjacent airborne routers are initially stored in the Loc-RIB of the air/ground router. These  routes may have been originated locally from local configuration information, or they may have been received from an adjacent ground routeur. At that stage the routes are defined with a security path attribute that reflects the security characteristics of the ground segment of the route only. Typically, the security path attribute of those routes does not comprise any Air/Ground subnetwork security tag. However it may comprise an ATSC class security Tag, indicating that the ground component of the route is available for ATSC traffic and whether non-ATSC traffic is allowed.

Before the advertisement, when copying those routes into the Adj-RIB-Out associated with an adjacent airborne BIS, an air-ground routeur is expected to update the Security Path Attribute of the routes taking into account the security characteristics of the mobile subnetworks that are currently supporting the adjacency with the peer airborne routeur. This is done as follows:

- For each Air/Ground Subnetwork supporting the adjacency between the Air/Ground routeur and the Airborne BIS, one A/G Subnetwork Type Security Tag Set is inserted, encoding the mobile subnetwork type (Mode S, VDL, AMSS, Gatelink or HF) and the restrictions on the traffic types that are not allowed to pass over  the subnetwork.

- If present, the value of ATSC Class Security Tag Set is downgraded to the ATSC Class being supported by the mobile adjacency;

- If all mobile subnetwork supporting the mobile adjacency are restricted to the support of ATSC-traffic only, then the name of the ATSC Class Security Tag Set (if present) is set to the hexadecimal value '07' so that to indicate that the route only supports ATSC traffic.

- On the other hand, if none of the mobile subnetworks supporting the adjacency are approved for ATSC traffic, then the ATSC Class Security Tag Set (if present) is removed form the Security Path Attribute.

The updated route is then advertised to the peer airborne BIS over one of the mobile subnetworks supporting the adjacency.

The Air/Ground BIS is tasked to re-update the Security Path attribute of the routes stored in the Adj-RIB-Out every time a change in the mobile subnetwork connectivity occurs over the adjacency with an Airborne routeur. For example, if a new subnetwork connection via a new mobile subnetwork becomes available to support the adjacency, then the air-ground BIS is expected to add a new Air/Ground Subnetwork Type Security Tag Set, and to possibly update the ATSC Class Security Tag Set (if present) of the route(s) advertised to the airborne router. Likewise, if a Leave event occurs for one of the subnetwork connectivity supporting the adjacency with the airborne routeur, then the air-ground BIS is expected to remove the corresponding Air/Ground Subnetwork Type Security Tag Set, and update the ATSC Class Security Tag Set of the route(s) advertised to the airborne router. The routes are then re-advertised to the adjacent ground routeurs. A change in the mobile subnetwork connectivity over the adjacency with an airborne routeur, triggers therefore the re-advertisement of the routes that are provided by the air/ground routeur to the airborne routeur.

3.4.10.3.7.3.2 Processing of uplinked routes by an airborne routeur

On receipt of an IDRP Update BISPDUs from an air/ground routeur, an airborne routeur does not need to update the security information of the route. The update has indeed been performed by the air/ground routeur. The airborne routeur is then simply expected to copy the route from the Adj-RIB-In to the Loc-RIB and to rebuild its FIB.

When a change in the mobile subnetwork connectivity occur, the role of the airborne routeur is also reduced to a minimum. The airborne routeur knows that this is the air/ground BIS which is tasked to update the security information of the routes taking into account the new characteristics of the mobile adjacency. The airborne routeur also knows that the updated routes will be re-advertised by the air/ground routeur. Hence, the airborne routeur simply needs to wait for the updated routes from the air/ground routeur.

3.4.10.3.7.3.2 Drawback of these baseline procedures

The re-advertisement of routes over air/ground adjacencies when mobile subnetwork connectivity changes was identified as a cumbersome behaviour: it incurs a waste of the bandwidth; and it may become a serious cost issue, especially when the re-advertised routes goes through a satellite subnetwork.

As an example, let us consider the case of an aircraft that has an adjacency with an Air/Ground Routeur over AMSS and also comes into contact with the same Air/Ground router through a VDL subnetwork, the A/G Router will re-advertise the routes that it had previously uplinked to the aircraft. These may be uplinked over either subnetwork. Should the aircraft then go out of contact with the VDL subnetwork then another re-advertisement of the routes is also expected, which then has to be over AMSS. This becomes a serious issue when the aircraft is in a VDL fringe area and might repeatedly go in and out of contact with the VDL subnetwork.

This issue has been resolved by the extension of the ATN Technical Provisions with the enhanced procedures for the exchange of routing information in the uplink direction (see the next section).

Air/Ground and the Airborne routeurs compliant with the most recent edition of the ATN Technical Provisions are expected to support these enhanced procedures. However, for backward compatibility reasons, these systems are also required to support and use the baseline procedures when faced to a peer routeur that does not support the enhanced procedures.

3.4.10.3.7.4 Enhanced procedures for the exchange of routing information in the uplink direction

Note: As explained before, the "enhanced" procedures'for the exchange of routing information in the uplink direction only applies over mobile adjacencies when both the airborne and the Air/Ground routeurs have indicated support of the enhanced capability to process Update BISPDU without Air/Ground Subnetwork Security Tag.

3.4.10.3.7.4.1 Update of the Security Path Attribute by an Air-Ground routeur

When the use of the enhanced procedures is supported over a mobile adjacency, the air/ground routeur is relieved of its duty to update the security information of the routes before their advertisement to the peer airborne routeur.

When a route is selected for advertisement to the peer Airborne routeur, the air/ground routeur simply needs to copy the route from the Loc-RIB to the relevant Adj-RIB-Out. The air/ground routeur is not expected to insert any A/G Subnetwork Type Security Tags in the Security Path Attribute of the route. The ATSC Class Security Tag (if present) must not be updated. The route is advertised to the airborne routeur, with a Security Path Attribute that holds information only on the security characteristics of the ground component of the route.

The routing information that is provided by an air/ground routeur to an airborne routeur is therefore totally independent from the characteristics of mobile subnetworks supporting the adjacency. When a change in the mobile subnetwork connectivity occurs over the adjacency with an airborne routeur, that routing information does not need to be updated. The Air/Ground routeur is therefore not expected to re-advertise the information every time the subnetwork connectivity changes. This is only when the security characteristics of the ground component of the uplinked routes are modified, that the Air/Ground routeur needs to update and re-advertise the routes.

3.4.10.3.7.3.2 Processing of uplinked routes by an airborne routeur

When the use of the enhanced procedures is supported over a mobile adjacency, the update of the security information of uplinked routes falls within the responsibility of the airborne routeur.

On receipt of an IDRP Update BISPDUs from an air/ground routeur, an airborne routeur will typically perform the update of the Security Path Attributes when it copies the route from the Adj-RIB-In where the received route has been stored to its Loc-RIB. The update rules are the same as the ones that would have been applied by the air/ground routeur, should the baseline procedures have been used over the adjacency. More specifically:

- For each Air/Ground Subnetwork supporting the adjacency between the Air/Ground routeur and the Airborne BIS, one A/G Subnetwork Type Security Tag Set is inserted, encoding the mobile subnetwork type (Mode S, VDL, AMSS, Gatelink or HF) and the restrictions on the traffic types that are not allowed to pass over  the subnetwork.

- If present, the value of the ATSC Class Security Tag Set is downgraded to the ATSC Class being supported by the mobile adjacency;

- If all mobile subnetwork supporting the mobile adjacency are restricted to the support of ATSC-traffic only, then the name of the ATSC Class Security Tag Set (if present) is set to the hexadecimal value '07' so that to indicate that the route only supports ATSC traffic.

- On the other hand, if none of the mobile subnetworks supporting the adjacency are approved for ATSC traffic, then the ATSC Class Security Tag Set (if present) is removed form the Security Path Attribute.

When a change in the mobile subnetwork connectivity occurs over the adjacency with an air/ground routeur, the airborne routeur is responsible to update in the Loc-RIB the security information of the routes received over this adjacency. For example, if a new subnetwork connection via a new mobile subnetwork becomes available to support the adjacency, then the airborne BIS is expected to add a new Air/Ground Subnetwork Type Security Tag Set, and to possibly update the ATSC Class Security Tag Set (if present) of the route(s) received over the adjacency. Likewise, if a Leave event occurs for one of the subnetwork connectivity supporting the adjacency with the air/ground routeur, then the airborne routeur is expected to remove the corresponding Air/Ground Subnetwork Type Security Tag Set, and update the ATSC Class Security Tag Set of the route(s) advertised to the airborne router.

3.4.10.4 Air-Ground Route Initiation without IDRP