ATNP/SGB1/WP127

Inter-domain BIS-BIS adjacency

Prepared by Christine Ricci

(France)

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

SGB1/1

Honolulu, Hawaii

27-28 Feb. 2001

SUMMARY

SG-B1/WP122 describes a means to forward NPDUs between BISs that belong to two adjacent routing domains and are not connected to the same subnetwork. Although IDG/4 agreed that this kind of architecture can be valid, we think it is in contradiction with the ISO 10747 standard.

TABLE OF CONTENTS

1	Background	i

2	Use of encapsulation in IDRP	i

3	Recommendation	ii

# Background

WP122 presents the possibility to interconnect ATN routing domains via BISs that are not located on the same subnetwork, or even via ISs. This possibility has been presented by ARINC and discussed during the IDG/4 meeting. IDG/4 agreed that ARINC’s proposed architecture was valid and that it is okay to encapsulate ATN CLNP within COTS CLNP to pass through routers that are not ATN compliant. However, the ISO 10747 stipulates that the interconnection between two adjacent routing domains must be done via two BISs connected to the same subnetwork. Moreover there is no requirement in the ATN SARPs or in the ISO/IEC 10747 standard to support the encapsulation mechanisms described in section 8.4 item b1) of ISO 10747. These mechanisms are supposed to be used only for intra-domain relaying of CLNP PDUs between BISs of the same routing domain. This paper will explain this mechanism.

# Use of encapsulation in IDRP

ISO 10747 defines adjacent routing domains as (quote from ISO10747):

“3.6.6: Adjacent RDs: Two RDs (“A” and “B”) are adjacent to one another if there is at least one pair of BISs, one located in “A” and the other in “B” that are attached to each other by means of a real subnetwork”

Moreover, section 5.9.2 of ISO 10747 specifies that (quote from ISO10747):

“5.9.2 External neighbor BIS

Each BIS may establish and maintain communications with other BISs in adjacent routing domains. A BIS has no direct communications link with any BIS in another routing domain unless that RD is adjacent to it, as defined in 3.6: that is, a BIS does not communicate directly with a BIS located in a different routing domain unless the pair of BISs are attached to at least one common subnetwork.”

That means that neighbour BISs in different routing domains must be attached to the same subnetwork. The only possibility to define neighbour BISs that are not attached to the same subnetwork is then related to intra-domain exchanges. Figure 1 describes this possibility:

IDRP

RD “B”

IDRP

BIS1

BIS2

BIS3

RD “A”

Figure 1

Section 8.4 of ISO 10747 describes the mechanisms by which CLNP PDUs are to be forwarded between BISs that are either located on the same subnetwork (which is the case for adjacent BISs of different routing domains) or on different subnetworks (which may be the case only for adjacent BISs of the same routing domain). Note e) in Table 1 of ISO/IEC 10747 clarifies that use of 8.4 items b1 and b2 for intra-domain forwarding.

In order not to be constrained by section 3.6.6 of ISO 10747, WP122 seems to imply that two routing domains that are connected via an intra-domain IS are not adjacent. In that case, it seems impossible that two BISs of those two routing domains are considered as IDRP neighbour BISs, as the IDRP standard recognises only two types of IDRP neighbours: Internal neighbour BIS and external neighbour BIS, with the adjacency rules specified in 5.9.2.

# Recommendation

Working Group B is requested not to insert the Guidance Material proposed in SG-B1/WP122 as the encapsulation mechanisms described in ISO 10747 section 8.4 b) relate to intra-domain forwarding of CLNP PDUs, which is out of the scope of the ATN SARPs.