|  | ATNP/WGB/WP 02- 26th February 2001 |
| --- | --- |

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP B (ATN COMMUNICATION SERVICES)

Honolulu, Hawaii, 26st February– 3rd March 2001

B2 - Infrastructure Communication Services

Proposed Corrections 
in ATN Secured Upper Layers

Prepared by: G. Mittaux-Biron

Presented by: A. Dedryvère

SUMMARY

This paper proposes modifications in SV 4 in order to correct defects, in terms of remaining typos and accepted PDRs (PDRM0090001 and PDRM0090002).

# Various typos:

## Change 4.3.3.7.4.2.3

4.3.3.7.4.2.3.When a SA-SEND Indication primitive is validly invoked, the CF is the receiverresponder CF, and is in the ASSOCIATION PENDING state, the CF shall:

## Change 4.3.3.7.4.2.7

4.3.3.7.4.2.7. When an SA-SEND Indication primitive is validly invoked, the CF is the release receiverresponder in the RELEASE PENDING state, the CF shall:

## Change 4.8.5.2.2.6.1

a)	retrieve, by locally-defined means, the identifier of the algorithm to be used by the SSO for checkinggenerating the appendix:

## Change 4.3.3.7.3.2.2

Add the following:

c)	Construct a SA-END Request primitive with the following parameters:

Table 4.3-40.

| SA-END Request parameter | ATN value |
| --- | --- |
| Calling Entity ID | as retrieved in a) above |
| Called Entity ID | as retrieved in a) above |
| Severity | Abstract value “low” |

d)	Invoke the SA-END Request primitive

# PDR M0090001

## Dialogue establishment phase

### AC-ASSOCIATE Request following a SEND-SEI Request

The following two changes split the AC-ASSOCIATE Request following a SEND-SEI Request: one manages the Key Management case, while the other manages the Secured Dialogue case. The Main difference between them is that the AC-ASSOCIATE Request User Information parameter is used in the former and that it is left unused in the latter.

#### Change 4.3.3.8.1.2.2

4.3.3.8.1.2.2 When a SEND-SEI Request pseudo-primitive is validly invoked on a Dialogue supporting key management, and the CF is in the ASSOCIATION PENDING state, and it is the Initiator CF, it shall:

#### Add 4.3.3.8.1.2.3

4.3.3.8.1.2.3	When a SEND-SEI Request pseudo-primitive is validly invoked on a Dialogue supporting secured exchanges, and the CF is in the ASSOCIATION PENDING state, and it is the Initiator CF, it shall:

a)	Determine the app-type as defined for the ATN-App AE,

b)	Construct the Application Context name, with the value of the "version" arc set equal to the DS-User Version Number parameter if provided, and set to zero otherwise,

c)	If not specified in the request primitive, retrieve the local Calling Presentation Address,

d)	Determine the Called Presentation Address either directly from the Called Presentation Address parameter if present, or via look-up from the Called Peer ID and Called Sys-ID parameters,

e)	If the Calling Peer Id parameter is present, then retrieve the corresponding Calling AP Title.  If, in addition to Calling Peer ID, the optional Calling Sys-ID parameter is present, then retrieve the corresponding Calling AE-Qualifier.  If Calling Peer ID is not present, then Calling AP Title and Calling AE-Qualifier are not used in the A-ASSOCIATE request (and they will not then be included in the resulting A-ASSOCIATE-REQUEST (AARQ) APDU),

Note. — The way that the Calling AP Title and the Calling AE-Qualifier are retrieved is a local implementation matter.

f)	Construct an A-ASSOCIATE Request primitive with the following parameters:

Table 4.3-52.

| A-ASSOCIATE Request parameter | ISO Status | ATN value |
| --- | --- | --- |
| Mode | U | Not used (default value) |
| Application Context Name | M | As derived in b) above |
| Application Context Name List | C | Not used |
| Calling AP Title | U | As derived in e) above |
| Calling AE Qualifier | U | As derived in e) above |
| Calling AP Invocation-identifier | U | Not used |
| Calling AE Invocation-identifier | U | Not used |
| Called AP Title | U | Not used |
| Called AE Qualifier | U | Not used |
| Called AP Invocation-identifier | U | Not used |
| Called AE Invocation-identifier | U | Not used |
| ACSE Requirements | U | Symbolic value “authentication” |
| Authentication-mechanism-name | U | Not used |
| Authentication-value | U | SESE PDU transmitted by the S-ASO via the pseudo primitive SEND-SEI Request |
| User Information | U | Not used |
| Calling Presentation Address | M | Derived as in c) above |
| Called Presentation Address | M | Derived as in d) above |
| Presentation Context Definition List | U | Not used |
| Default Presentation Context Name | U | Not used |
| Quality of Service | M | See 4.3.3.3.2.3 |
| Presentation Requirements | U | Not used (default value) |
| Session Requirements | M | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | C | Not used |
| Initial Assignment of Tokens | C | Not used |
| Session-connection Identifier | U | Not used |

g)	Invoke the A-ASSOCIATE Request primitive

h)	Enter the ASSOCIATION PENDING state as an initiator CF.

### AC-ASSOCIATE Indication

This change allows the provision of both the SESE PDU and the optionally empty contents of the AC-ASSOCIATE Indication User Information parameter

#### Change 4.3.3.4.1.2.2 c)

4.3.3.4.1.2.2. When an A-ASSOCIATE Indication primitive is validly invoked with the ACSE Requirements and the Calling Peer Id parameters both present, the CF shall:

…

c) Provide the SESE PDU and the contents of the ACSE User Information parameter to the Security ASO using the SEND-SEI indication pseudo primitive (see 4.8)

### SA-SEND Indication

This change corrects an error in table 4.3-42 where the User Data was expected to be in the AC-ASSOCIATE Indication User Information parameter. In fact, on an SA-SEND Indication primitive activation, the User Data are always in the SA-SEND Indication User Data parameter.

#### Change Table 4.3-42

Table 4.3-42.

| D-START Indication parameter | Value |
| --- | --- |
| Calling Peer ID | Derived as in b) above |
| Calling Sys-ID | Derived as in b) above |
| Calling Presentation Address | Derived as in b) above |
| DS-User Version Number | Derived as in a) above |
| Security Requirements | Abstract value “Secured Dialogue” |
| Quality Of Service | See 4.3.3.4.1.3. |
| User Data | A-ASSOCIATE User InformationSA-SEND Indication User Data parameter |

### AC-ASSOCIATE Response following a SEND-SEI Request

The following two changes split the AC-ASSOCIATE Response following a SEND-SEI Request: one manages the Key Management case, while the other manages the Secured Dialogue case. The Main difference between them is that the AC-ASSOCIATE Response User Information parameter is used in the former and that it is left unused in the latter.

#### Change 4.3.3.8.1.2.3

4.3.3.8.1.2.3	When a SEND-SEI Request pseudo-primitive is validly invoked on a Dialogue supporting key management, and the D-START Result parameter had the abstract value “accepted», «rejected (permanent)” or “rejected (transient)”, and the CF is in the ASSOCIATION PENDING state, and it is the Responder CF, it shall:

#### Add 4.3.3.8.1.2.5

4.3.3.8.1.2.5	When a SEND-SEI Request pseudo-primitive is validly invoked on a Dialogue supporting secured exchanges, and the D-START Result parameter had the abstract value “accepted”, “rejected (permanent)” or “rejected (transient)”, and the CF is in the ASSOCIATION PENDING state, and it is the Responder CF, it shall:

a)	Construct the Application Context name, with the value of the "version" arc set equal to the DS-User Version Number parameter if provided, and set to zero otherwise,

b)	Retrieve the responding Presentation Address,

c)	Construct an A-ASSOCIATE Response primitive with the following parameters:

Table 4.3-54.

| A-ASSOCIATE Response parameter | ISO Status | ATN Value |
| --- | --- | --- |
| Application Context Name | M | As derived in a) above |
| Application Context Name List | C | Not used |
| Responding AP Title | U | Not used |
| Responding AE Qualifier | U | Not used |
| Responding AP Invocation-identifier | U | Not used |
| Responding AE Invocation-identifier | U | Not used |
| ACSE Requirements | C | Symbolic value “authentication” |
| Authentication-mechanism-name | U | Not used |
| Authentication-value | U | SESE PDU transmitted by the S-ASO via the pseudo primitive SEND-SEI Request |
| User Information | U | Not used |
| Result | M | D-START Result parameter |
| Diagnostic | U | Not used |
| Responding Presentation Address | M | Derived as in b) above |
| Presentation Context Definition Result List | C | Not used |
| Default Presentation Context Result | C | Not used |
| Quality of Service | M | See 4.3.3.3.2.3 |
| Presentation Requirements | U | Not used (default value) |
| Session Requirements | M | No Orderly Release (NOR), Duplex |
| Initial Synchronization Point Serial No | C | Not used |
| Initial Assignment of Tokens | C | Not used |
| Session-connection Identifier | U | Not used |

d)	If the D-START Response Result parameter has the abstract value “accepted”, invoke an A-ASSOCIATE Response primitive with the Result parameter set to “accepted”, and remain in the ASSOCIATION PENDING state.

e)	If the D-START Response Result parameter has the abstract value “rejected (permanent)” or “rejected (transient)”, invoke an A-ASSOCIATE Response primitive with the Result parameter set to the same abstract value, and remain in the ASSOCIATION PENDING state.

### AC-ASSOCIATE Confirmation

This change allows the provision of both the SESE PDU and the optionally empty contents of the AC-ASSOCIATE Confirmation User Information parameter

#### Change 4.3.3.4.2.2.3 b)

4.3.3.4.2.2.3. When an A-ASSOCIATE Confirmation primitive is validly invoked on a Dialogue for which security was initially required, with a valid ACSE Authentication Value parameter, the CF shall:

…

b) Provide the SESE PDU and the contents of the ACSE User Information parameter to the Security ASO using the SEND-SEI indication pseudo primitive (see 4.8)

### SA-SEND Indication

These changes correct two errors in table 4.3-39 and in 4.3-41 where the User Data was expected to be in the AC-ASSOCIATE Confirmation User Information parameter. In fact, on an SA-SEND Indication primitive activation, the User Data are always in the SA-SEND Indication User Data parameter.

#### Change Table 4.3-39

Table 4.3-39.

| D-START Confirmation parameter | Value |
| --- | --- |
| DS-User Version Number | Derived as in a) above |
| Security Requirements | Abstract value “Secured Dialogue” |
| Quality Of Service | See 4.3.3.4.1.3 |
| Result | “accepted” |
| Reject Source | Not used |
| User Data | A-ASSOCIATE User InformationSA-SEND Indication User Data parameter |

#### Change Table 4.3-41

Table 4.3-41.

| D-START Confirmation parameter | Value |
| --- | --- |
| DS-User Version Number | Derived as in c) above |
| Security Requirements | Abstract value “Secured Dialogue” |
| Quality Of Service | See 4.3.3.4.1.3. |
| Result | “rejected (permanent)” or “rejected (transient)”, from the A-ASSOCIATE Result parameter |
| Reject Source | Derived as in d) above |
| User Data | A-ASSOCIATE User InformationSA-SEND Indication User Data parameter |

#### Change Table 4.3-44

Table 4.3-44.

| D-START Confirmation parameter | Value |
| --- | --- |
| DS-User Version Number | Derived as in c) above |
| Security Requirements | Abstract value “Secured Dialogue” |
| Quality Of Service | See 4.3.3.4.1.3. |
| Result | “rejected (permanent)” or “rejected (transient)”, from the A-ASSOCIATE Result parameter |
| Reject Source | Derived as in d) above |
| User Data | A-ASSOCIATE User InformationSA-SEND Indication User Data parameter |

## Data transfer phase

This modification clarifies the use of SA-SEND Indication user data parameter as D-DATA Indication user data.

### Change in 4.3.3.7.4.2.5

4.3.3.7.4.2.5 When a SA-SEND Indication primitive is validly invoked, the CF is in the DATA TRANSFER state, and is not aborting, the CF shall issue a D-DATA Indication primitive to the DS-User, with the received data value SA-SEND Indication user data parameter as the user data parameter, and remain in the same state.

## Dialogue release phase

This modification corrects the use of SA-SEND Indication user data parameter as D-END user data instead of A-RELEASE user data parameters.

### Change in 3.3.7.4.2.7

a)	Construct a D-END Indication primitive, with the User Data parameter set equal to the value of the User Information Data parameter of the A-RELEASE SA-SEND Indication primitive,

### Change in 3.3.7.4.2.8

Table 4.3-47.

| D-END Confirmation parameter | Value |
| --- | --- |
| Result | “affirmative” |
| User Data | User Information Data parameter from the A-RELEASE Confirmation SA-SEND Indication, if present |

### Change in 3.3.7.4.2.9

Table 4.3-48.

| D-END Confirmation parameter | Value |
| --- | --- |
| Result | “rejected” |
| User Data | User Information Data parameter from the A-RELEASE Confirmation SA-SEND Indication, if present |

## Dialogue abort phase

This modification corrects the use of SA-SEND Indication user data parameter as D-DATA Indication user data instead of A-ABORT Indication user data parameter.

### Change in 3.3.7.4.2.6

d)	If the Abort Source parameter of the A-ABORT Indication is set to “ACSE service-user” and the Diagnostic parameter is set to “No reason given”, issue a D-ABORT Indication primitive to the DS-User, with the Originator parameter set to “User” and the User Data parameter set equal to the User Information parameter in the A-ABORT Indication SA-SEND Indication user data parameter, if present.

e)	If the Abort Source parameter of the A-ABORT Indication is set to “ACSE service-user” and the Diagnostic parameter is absent or is set to any value other than “No reason given”, then issue a D-ABORT Indication primitive to the DS-User, with the Originator parameter set to “Provider” and the User Data parameter set equal to the User Information parameter in the A-ABORT Indication SA-SEND Indication user data parameter, if present.

f)	If the Abort Source parameter of the A-ABORT Indication has the abstract value “ACSE service-provider”, then issue a D-ABORT Indication primitive to the DS-User, with the Originator parameter set to the abstract value “Provider”, and the User Data parameter set equal to the User Information parameter in the A-ABORT Indication SA-SEND Indication user data parameter, if present.

# Calling Peer ID

## Change in 4.3.3.3.2.2.2

4.3.3.3.2.2.2 When the D-START Request is validly invoked with the Security Requirements parameter set to the abstract value “Secured Dialogue Supporting Key Management”, and the Calling Peer Id parameter is present, the CF shall:

## Change in 4.3.3.3.2.2.3

4.3.3.3.2.2.3 When the D-START Request is validly invoked with the Security Requirements parameter set to the abstract value “Secured Dialogue”, and the Calling Peer Id parameter is present, the CF shall:

## Change in 4.3.3.8.1.2.2 e)

e)	Retrieve the Calling AP Title from the Calling Peer Id, or, if absent, by local knowledge, from the value of the ICAO Facility Designator affected to the Dialogue user. If there is no ICAO Facility Designator affected to the Dialogue user, then the default abstract value ATN-DefaultAPTitle should be used.  If the optional Calling Sys-ID parameter is present, then retrieve the corresponding Calling AE-Qualifier.  If the optional parameter Calling Sys-ID is not present, then Calling AE-Qualifier is not used in the A-ASSOCIATE request (and it will not then be included in the resulting A-ASSOCIATE-REQUEST (AARQ) APDU),

# Retrieval of the algorithm identifier for checking appendixes

The SV 8 document uses the “value” field of the ATNAppendix ASN.1 description for identification of the algorithm identifier used in computing appendixes. For the recall, the ASN.1 description of the ATNAppendix is given as follows:

ATNAppendix ::= SEQUENCE {

algorithmId	AlgorithmIdentifier OPTIONAL,

validity		CHOICE {

timeField	ATNSecurityDateTime ,

random		INTEGER (0..4294967295),

-- 32-bit unsigned integer

counter		INTEGER (0..MAX),

-- unsigned, unconstrained integer

...  } OPTIONAL,

value		CHOICE {

ecdsa-Signature		ECDSA-Sig-Value,

hmac-Tag		OCTET STRING (SIZE (4, ...)),

...  }

}

## SE-TRANSFER Indication

These changes remove the description of the retrieval of the algorithm identifier and its provision to the SSO-SignCheck function when managing the SE-TRANSFER Indication event.

### Change 4.8.5.2.2.2.1

a)	retrieve, by locally-defined means, the identifier of the algorithm to be used by the SSO for checking the appendix:

1)	If no algorithm identifier is locally-defined and if the algorithmIdentifier field of the atnSignature field of the SE-TRANSFER Indication Security Exchange Item parameter is present, then use its associated value as the identifier of the algorithm to be used by the SSO,

2)	If no algorithm identifier is locally-defined and if the algorithmIdentifier field of the atnSignature field of the SE-TRANSFER Indication Security Exchange Item parameter is not present then, unless there is prior agreement otherwise, use the abstract value “Signature-appendix” as the identifier of the algorithm to be used by the SSO.

### Change Table 4.8-8

Table 4.8-8.

| SSO-SignCheck function parameter | ATN value |
| --- | --- |
| Source Peer | Calling EntityID |
| Destination Peer | Called EntityID |
| Algorithm identifier | set to the value retrieved in a) |
| Certificate Path | set to the value retrieved in ba) |
| User Data | received User Data |
| Security Item | atnSignature field of the SE-TRANSFER Indication Security Exchange Item parameter. |

### Change 4.8.5.2.2.2.2

a)	retrieve, by locally-defined means, the identifier of the algorithm to be used by the SSO for checking the appendix:

1)	If no algorithm identifier is locally-defined and if the algorithmIdentifier field of the atnSignature field of the SE-TRANSFER Indication Security Exchange Item parameter is present, then use its associated value as the identifier of the algorithm to be used by the SSO,

2)	If no algorithm identifier is locally-defined and if the algorithmIdentifier field of the atnSignature field of the SE-TRANSFER Indication Security Exchange Item parameter is not present then, unless there is prior agreement otherwise, use the abstract value "”MAC-appendix”" as the identifier of the algorithm to be used by the SSO.

### Change Table 4.8-9

Table 4.8-9.

| SSO-SignCheck function parameter | ATN value |
| --- | --- |
| Source Peer | Called EntityID |
| Destination Peer | Calling EntityID |
| Algorithm identifier | set to the value retrieved in a) |
| Certificate Path | Not usedset to the value retrieved in a) |
| User Data | received User Data |
| Security Item | atnSignature field of the SE-TRANSFER Indication Security Exchange Item parameter. |

### Change 4.8.5.2.2.2.3

a)	retrieve, by locally-defined means, the identifier of the algorithm to be used by the SSO for checking the appendix:

1)	If no algorithm identifier is locally-defined and if the algorithmIdentifier field of the atnSignature field of the SE-TRANSFER Indication Security Exchange Item parameter is present, then use its associated value as the identifier of the algorithm to be used by the SSO,

2)	If no algorithm identifier is locally-defined and if the algorithmIdentifier field of the atnSignature field of the SE-TRANSFER Indication Security Exchange Item parameter is not present, and if both the Calling EntityID and the Called EntityID refer to ground entities then, unless there is prior agreement otherwise, use the abstract value “Signature-appendix” as the identifier of the algorithm to be used by the SSO.

3)	If no algorithm identifier is locally-defined and if the algorithmIdentifier field of the atnSignature field of the SE-TRANSFER Indication Security Exchange Item parameter is not present, and if either Calling EntityID or Called EntityID refers to an airborne entity then, unless there is prior agreement otherwise, use the abstract value “MAC-appendix” as the identifier of the algorithm to be used by the SSO.

### Change Table 4.8-10

Table 4.8-10.

| SSO-ProtectSignCheck function parameter | ATN value |
| --- | --- |
| Source Peer | Remote EntityID |
| Destination Peer | Local EntityID |
| Algorithm identifier | set to the value retrieved in a) |
| Security Item | atnProtectSign field of the SE-TRANSFER Indication Security Exchange Item parameter. |

# Retrieval of the certificate paths

The last changes in the structure describing the certificates, which move to a SEQUENCE OF ATNCertificates have not been correctly taken into account.

## SE-TRANSFER Indication

### Change 4.8.5.2.2.2.1

b)	if the signature key certificate path of the remote system is contained in the atnCertificates field of the ATNEstablish part of the SE-TRANSFER Indication Security Exchange Item parameter is present, then extract from it the signature key certificate of the peer; otherwise it is not used.

c)	if the key agreement public key certificate  of the remote system is contained in the atnCertificates field of the ATNEstablish part of the SE-TRANSFER Indication Security Exchange Item parameter is present, then extract from it the key agreement key certificate of the peer; otherwise it is not used.

b)	retrieve the certificates of the remote system from the atnCertificates field of the ATNEstablish part of the SE-TRANSFER Indication Security Exchange Item parameter.