ATNP WG-B/SG-B3

WP-0408A

Aeronautical Telecommunications Network Panel (ATNP)

Working Group B

Security Sub-group SG-B3

Meeting 4

Atlantic City, NJ

14-16 January 2002

Working Paper

Supplemental Validation Report for the

Aeronautical Telecommunications Network (ATN)

Public Key Infrastructure (PKI) Laboratory

Prepared By: Honeywell Advanced Systems Technology Group

For

NASA Ames Research Center

SUMMARY

This report addresses the PKI Laboratory Validation program, which is one of many programs established to support the validation of the ATN Security Provisions, Document 9705, Sub-Volume VIII.  The purpose of this document is to report the results of on-going validation activities completed during calendar year 2001.

DOCUMENT REVISION LOG

| Revision | Description | Date |
| --- | --- | --- |
| 1.0 | Distribution to WG-B/SG-B3 at Meeting 4 in Toulouse, France (Meeting Cancelled) | 25-26 September 2001 |
| 1.1 | Distribution to WG-B/SG-B3 at Meeting 4 in Atlantic City, NJ | 14-16 January 2002 |

TABLE OF CONTENTS

1.	Introduction	1

1.1.	Document Scope	1

1.2.	Document Overview	1

1.3.	Reference Documents	2

1.4.	Dependencies on External Standards	2

1.4.1.	PKI	2

1.4.2.	ASN.1/PER	2

1.5.	Terminology	2

2.	PKI Laboratory Validation Obectives	3

2.1.	Relationship to ATN Validation Objectives	3

2.2.	Relationship to SARPS Requirements	5

2.3.	High-Level PKI Laboratory Objectives	5

Table 2. Validation Objectives and Means of PKI Laboratory Validation Program	6

Table 3.  Validation Means	6

3.	PKI Laboratory Architecture	7

4.	PKI Laboratory Validation Strategy	9

4.1.	Overall Strategy	9

4.2.	Assumptions and Constraints	9

4.3.	Test Scenarios	9

4.3.1.	PKI Laboratory Independent Test Scenarios	10

4.3.1.1.	ATN PKI Certificates	10

4.3.1.1.1.	Test Scenarios	10

4.3.1.1.2.	Expected Output	10

5.	PKI Laboratory Environment	15

5.1.1.	Entrust CA/RA	15

5.1.1.1.	Description	15

5.1.1.2.	Environment—Entrust CA	15

5.1.1.3.	Environment—Entrust RA	15

5.1.2.	Xcert Sentry CA	15

5.1.2.1.	Description	15

5.1.2.2.	Environment	15

5.1.3.	UniCERT CA	15

5.1.3.1.	Description	15

5.1.3.2.	Environment	16

5.1.4.	Certicom Trustpoint	16

5.1.4.1.	Description	16

5.1.4.2.	Environment	16

6.	Detailed Analysis and Defect Report	17

6.1.	Entrust PKI 5.1.1	17

6.2.	RSA (Xcert) Sentry CA	17

6.3.	Baltimore UniCERT	17

6.4.	CERTICOM Trustpoint	17

Figure 3. Detailed Analysis of RSA (formerly Xcert) SentryCA Certificate	19

Figure 4. Detailed Analysis of Certicom Trustpoint Certificate	23

7.	RESUlts and analysis	25

7.1.	FVO 6	25

7.2.	TVO 8	26

8.	Conclusions	27

Supplemental Validation Report for the Aeronautical Telecommunications Network (ATN) Public Key Infrastructure (PKI) Laboratory

# Introduction

## Document Scope

This report addresses the PKI Laboratory Validation program, which is one of many programs established to support the validation of the ATN Security Provisions, Document 9705, Sub-Volume VIII.  The purpose of this document is to report the results of on-going validation activities completed during calendar year 2001.  This report is intended to supplement Validation Report WP-2106, presented during Meeting 20 of WG-1/SG-2 in Berlin, Germany, 21-22 August 2000.

## Document Overview

This document is organized as follows:

- Section 1 – Introduction

The introduction identifies the scope of the document, summarizes the document organization, provides references to applicable documents, identifies dependencies on external standards, and defines terminology used throughout the document.

- Section 2 – PKI Laboratory Validation Objectives

This section documents the relationship to high-level ATN validation objectives, describes the high-level PKI Laboratory objectives, and provides a cross-reference to specific SARPs requirements.

- Section 3 – PKI Laboratory Architecture

This section describes the architecture of the PKI Laboratory with respect to the ATN communications environment.

- Section 4 – PKI Laboratory Validation Strategy

This section defines the strategy for achieving the validation objectives, including assumptions, constraints and test scenarios.

- Section 5 – PKI Laboratory Environment

This section describes the PKI Laboratory environment including tools and techniques.

- Section 6 – Detailed Analysis and Defect Report

This section documents the detailed analysis and any defects uncovered during execution of this validation exercise.

- Section 7 – Results and Analysis

This section contains a summary of results with respect to the PKI Laboratory validation objectives.

- Section 8 – Conclusions

This section provides a summary conclusion of the PKI Laboratory validation.

## Reference Documents

Draft ATN SARPs Technical Provisions, ICAO Document 9705 Sub-Volume VIII, W1S2WP2109.

Appendix I, Doc 9705 – Sub-Volume VIII (Security), Validation Report, ATNP/3

## Dependencies on External Standards

### PKI

- ITU-T X.509

### ASN.1/PER

- ISO 8825-2

## Terminology

| ASN | Abstract Syntax Notation |
| --- | --- |
| ATN | Aeronautical Telecommunications Network |
| ATNP | Aeronautical Telecommunications Network Panel |
| CA | Certificate Authority |
| CM | Context Management |
| COTS | Commercial Off-The-Shelf |
| CRL | Certificate Revocation List |
| FVO | Functional Validation Objective |
| ICAO | International Civil Aviation Organization |
| IDRP | Inter-Domain Routing Protocol |
| PER | Packed Encoding Rules |
| PKI | Public Key Infrastructure |
| SARPs | Standards and Recommended Practices |
| SVO | System Validation Objective |
| TVO | Technical Validation Objective |
| ULCS | Upper Layer Communication Services |

# PKI Laboratory Validation Obectives

## Relationship to ATN Validation Objectives

The primary goal of the PKI Laboratory Validation program is to address the PKI portion of several Sub-Volume VIII Validation Objectives by conducting test scenarios defined in this document and documenting the objective, measurable results.  For the on-going validation effort, the primary Validation Objectives include FVO6 and TV08 as defined in Table 1.  The ongoing PKI Laboratory Validation program does not address the remaining Validation Objectives.

Table 1.  High-level Sub-Volume VIII Validation Objectives Allocated to the PKI Laboratory Program

| Sub-Volume VIII Validation Objective | Description | Validation Objective Allocated to PKI Laboratory Program |
| --- | --- | --- |
| SVO 1 | To determine which system level requirements are satisfied by the Sub-Volume VIII requirements. | None. |
| SVO 2 | Validate that the Sub-Volume VIII requirements trace to other SARPs sub-volumes, where applicable. | None. |
| SVO 3 | Validate that Sub-Volume VIII includes provision for backward compatibility with prior versions of peer ATN implementations that do not incorporate security services. | None. |
| FVO 1 | Validate that Sub-Volume VIII supports implementation of local security policies and practices, within the boundaries of SARPs, as determined by States/Organizations. | Validate that the SARPs supports implementation of local PKI policies and practices, within the boundaries of the SARPs, as determined by States/Organizations. |
| FVO 2 | Validate that Sub-Volume VIII requirements are complete. | Validate that the Sub-Volume VIII PKI requirements are complete. |
| FVO 3 | Validate that Sub-Volume VIII requirements are unambiguous. | Validate that the Sub-Volume VIII PKI requirements are unambiguous. |
| FVO 4 | Validate that Sub-Volume VIII requirements are consistent. | Validate that the Sub-Volume VIII PKI requirements are consistent. |
| FVO 5 | Determine if there are any Sub-Volume VIII requirements that would have no effect if removed. Note: Interpret this VO to mean that there are no requirements in Sub-Volume VIII that are not necessary for the intended functionality, or to achieve migration to future versions. It is not meant to eliminate possible duplicate statements of requirements. | Determine if there are Sub-Volume VIII PKI requirements that would have no effect if removed. Note: Interpret this VO to mean that there are no requirements in Sub-Volume VIII that are not necessary for the intended functionality, or to achieve migration to future versions. It is not meant to eliminate possible duplicate statements of requirements. |
| FVO 6 | To determine if provision has been made to ensure that Sub-Volume VIII are implementation independent. | To determine if provision has been made to ensure that Sub-Volume VIII PKI requirements are implementation independent. |
| FVO 7 | To determine if Sub-Volume VIII includes provision for security services necessary for all security users. | Validate that the Sub-Volume VIII includes provision for a PKI that supports security services necessary all security users. |
| TVO 1 | Validate that Sub-Volume VIII includes provisions for both mobile and fixed ATN users. | Validate that the Sub-Volume VIII includes provision for a PKI that supports both mobile and fixed ATN users. |
| TVO 2 | Validate that Sub-Volume VIII minimizes air-ground security related protocol overhead. | Validate that the Sub-Volume VIII includes provision for a PKI that supports minimal air-ground overhead to meet required security level. |
| TVO 3 | Validate that Sub-Volume VIII supports the security provisions of the ATN Upper Layer Communications Service (ULCS). | Validate that the Sub-Volume VIII includes provision for a PKI that supports the security provisions of the ATN Upper Layer Communication Services (ULCS). |
| TVO 4 | Validate that Sub-Volume VIII supports the security provisions of the ATN Inter-Domain Routing Protocol (IDRP). | Validate that the Sub-Volume VIII includes provision for a PKI that supports the security provisions of the ATN Inter-Domain Routing Protocol (IDRP). |
| TVO 5 | Validate that independent implementations built in accordance to Sub-Volume VIII will be able to interoperate. | Validate that independent implementations built in accordance to Sub-Volume VIII will be able to interoperate. |
| TVO 6 | To determine if the ATN security solution has any unacceptable behaviour. | To determine if the ATN security solution has any unacceptable behaviour. |
| TVO 7 | To determine if provision for future migration has been addressed. | To determine if provision for future migration has been addressed. |
| TVO 8 | To determine if the functionality described in Sub-Volume VIII is implementable. | To determine if the functionality described in Sub-Volume VIII is implementable. |

## Relationship to SARPS Requirements

The following individual Sub-Volume VIII requirements or complete sections of Sub-Volume VIII requirements are to be validated by the PKI Laboratory effort.

8.3.1.1.4	Singular requirement regarding Public Key Certificates

8.3.1.2 [all]	Key Management and Distribution

8.3.1.9 [all]	Role of ATN Directory Servers within the ATN Security Framework

8.4.1 [all]	Certificate Policy

8.4.2 [all]	Certificate Practice Statement

8.4.3.1 [all]	ATN PKI Certificate Format – General Format

8.4.3.2 [all]	ATN PKI Certificate Format – ATN Certificates to be Signed

8.4.3.3 [all]	ATN PKI Certificate Format – Compressed Certificates

8.4.3.4 [all]	ATN PKI Certificate Format – ATN Certificate Path

8.4.4 [all]	ATN PKI CRL Format

8.4.5 [all] 	ATN PKI Certificate and CRL Validation

8.4.6 [all]	ATN PKI ASN.1 Module

8.5.2 [all]	ATN Cryptographic Setting

## High-Level PKI Laboratory Objectives

Table 2 identifies individual Sub-Volume VIII requirements or complete sections of Sub-Volume VIII requirements cross-referenced to specific Validation Objectives that are to be addressed by the PKI Laboratory effort.  Table entries indicate the validation means, as defined in Table 3, achieved as part of this on-going validation effort.

## Table 2. Validation Objectives and Means of PKI Laboratory Validation Program

| Requirements Grouping | Sub-Volume VIII Requirements | SVO |  |  |  | FVO |  |  |  |  |  |  | TVO |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 |  | 2 | 3 | 4 | 5 | 6 | 7 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| ATN Security Strategy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Security Architecture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Security Backward Compatibility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ULCS Security Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CM Security Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Applications Security Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Key Management and Distribution | 8.3.1.2, 8.3.1.9 |  |  |  |  |  |  |  |  |  | g |  |  |  |  |  |  |  |  | g |
| ATN Certificate Authority Architecture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN PKI Certificates | 8.3.1.1.4, 8.4.1, 8.4.2, 8.4.3.1, 8.4.3.2, 8.4.6 |  |  |  |  |  |  |  |  |  | d |  |  |  |  |  |  |  |  | d |
| ATN Compressed Certificates | 8.4.3.3, 8.4.3.4 |  |  |  |  |  |  |  |  |  | g |  |  |  |  |  |  |  |  | g |
| ATN Certificate Revocation Lists | 8.4.4 |  |  |  |  |  |  |  |  |  | g |  |  |  |  |  |  |  |  | g |
| ATN Cryptographic Setting | 8.5.2 |  |  |  |  |  |  |  |  |  | d |  |  |  |  |  |  |  |  | d |
| ATN Key Agreement Scheme (AKAS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Digital Signature Scheme (ADSS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Keyed Message Authentication Code Scheme (AMACS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Auxiliary Cryptographic Primitives and Functions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Security Object (SSO) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Table 3.  Validation Means

| Code | Description |
| --- | --- |
| a | Two or more independently developed interoperating implementations validated by two or more states/organizations. |
| b | Two or more independently developed interoperating implementations validated by one state/organization. |
| c | One implementation validated by more than one state/organization. |
| d | One implementation validated by one state/organization. |
| e | Partial implementation validated by one state/organization. |
| f | Simulation, analysis using tools e.g. compiler, modeling tools. |
| g | Analysis and inspection. |

# PKI Laboratory Architecture

The Honeywell PKI Lab is shown in Figure 1, and consists of the following components: Entrust CA (Certificate Authority) and Entrust RA (Registration Authority) interconnected via local area network; standalone Xcert SentryCA (now known as RSA Bsafe); standalone UniCERT CA; and standalone Certicom TrustPoint CA.  The laboratory environment is described in Section 5 of this document.

Entrust CA

Entrust RA

PeerLogic

Directory Server

(co-hosted on RA)

Xcert SentryCA

UniCERT

Certicom Trustpoint CA

CA

x.500

Figure 1.  PKI Validation Laboratory PKI Architecture

This page intentionally left blank.

# PKI Laboratory Validation Strategy

## Overall Strategy

The overall strategy for the PKI Laboratory Validation effort is to validate selected ATN Validation Objectives and specific ATN Technical Provisions, ICAO Document 9705, Sub-Volume VIII requirements using Commercial Off the Shelf PKI Systems (FVO6).

## Assumptions and Constraints

TBD.

## Test Scenarios

The PKI Laboratory test scenarios include only those scenarios that can be preformed with the PKI Laboratory resources and personnel independently.

### PKI Laboratory Independent Test Scenarios

#### ATN PKI Certificates

##### Test Scenarios

Numerous test scenarios will be described in detail to address the generation of certificates within the boundaries of the SARPs requirements listed in Table 2.  The structure of an ATN Certificate is shown in Figure 2.

##### Expected Output

The results of these test scenarios will be electronic format and printed and human readable copies of certificates generated in accordance with the variables defined in the test scenario.  These electronic format and printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08). Any unacceptable behavior noted during the tests will be documented (FVO6).  The detailed analysis, including any defects, is included in Section 6 and results are documented in Section 7.

| Figure 2. ATN Certificate Structure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Title | ASN.1 Name | ASN.1 Production Sequence | Tag | Valid Values | Invalid Values |
| version |  | Version 3 | INTEGER | 2 | 0, 1 |
| serialNumber | CertificateSerialNumber |  | INTEGER | Any | Non-Integer |
| signature | Algorithm | ecdsa-with-SHA1 | OBJECT IDENTIFIER | 1.2.840.10045.4.1 | Other OIDs |
|  | Parameters | NULL | NULL | NULL | Non-NULL |
| issuer | AttributeType | X.501 Distinguished Name | OBJECT IDENTIFIER | X.501 Distinguished Name Types | Non-X.501 Distinguished Name |
|  | AttributeValue | ANY DEFINED BY AttributeType | Type Dependent | Type Dependent | Type Dependent |
| validity | NotBefore | Coordinated Universal Time to seconds | UTCTime | YYMMDDhhmmssZ | Non-YYMMDDhhmmssZ |
|  | NotAfter | Coordinated Universal Time to seconds | UTCTime | YYMMDDhhmmssZ (any time after notBefore) | YYMMDDhhmmssZ (any time equal to or before notBefore) |
| subject | AttributeType | X.501 distinguished name for CA or AMHS entity; Empty otherwise | OBJECT IDENTIFIER | X.501 Distinguished Name Types / Empty | Empty / Non-X.501 Distinguished Name Types |
|  | AttributeValue | ANY DEFINED BY AttributeType | Type Dependent | Type Dependent | Type Dependent |
| subjectPublicKeyInfo | Algorithm | Id-ecPublicKey | OBJECT IDENTIFIER | 1.2.840.10045.2.1 |  |
|  | Parameters | NamedCurve sect163r2 (for non-CAs) sect233r1 (for CAs) | OBJECT IDENTIFIER | 1.3.132.0.15 1.3.132.0.27 | Other OIDs |
|  | SubjectPublicKey | ECPoint | BIT STRING | 02¦¦xQ (compressed) 03¦¦xQ (compressed) 04¦¦xQ¦¦yQ (uncompressed) | Any other form |
| issuerUniqueID |  |  |  | Omitted | Any |
| subjectUniqueID |  |  |  | Omitted | Any |
| Extensions |  |  |  |  |  |
| AuthorityKeyIdentifier | ExtnID | id-ce-authorityKeyIdentifier | OBJECT IDENTIFIER | 2.5.29.35 | Other OIDs |
|  | Critical | Not Critical | BOOLEAN | FALSE | TRUE |
|  | ExtnValue - keyIdentifier |  | OCTET STRING | 0100 plus 60 least significant bits of the SHA-1 hash of the subjectPublicKey | Other than Valid Values |
|  | ExtnValue – authorityCertIssuer |  |  | Omitted | Any |
|  | ExtnValue - authorityCertSerialNumber |  |  | Omitted | Any |
| KeyUsage | extnID | id-ce-keyUsage | OBJECT IDENTIFIER | 2.5.29.15 | Other OIDs |
|  | critical | Non-critical | BOOLEAN | FALSE | TRUE |
|  | extnValue - Key Usage |  | BIT STRING digitalSignature (0) nonRepudiation (1) keyEncipherment (2) dataEncipherment (3) keyAgreement (4) keyCertSign (5) cRLSign (6) encipherOnly (7) decipherOnly (8) | 000000001 000010000 001100000 | All other combinations |
| SubjectAltName | extnID | id-ce-subjectAltName | OBJECT IDENTIFIER | 2.5.29.17 | Other OIDs |
|  | critical | Non-critical | BOOLEAN | FALSE | TRUE |
|  | extnValue - SubjectAltName | General Names | [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 3: AMHS Address (See Note) 7:NET OCTET STRING SIZE (20) (See Note) 8:AP-Title (See Note) | 0: 1: 2: 4: 5: 6: |
| IssuerAltName | extnID | id-ce-issuerAltName | OBJECT IDENTIFIER | 2.5.29.18 | Other OIDs |
|  | critical | Non-critical | BOOLEAN | FALSE | TRUE |
|  | extnValue - issuerAltName | General Names | CHOICE [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 8:AP-Title (See Note) | 0: 1: 2: 3: 4: 5: 6: 7: |
| Algorithm OID | algorithm | ecdsa-with-SHA1 | OBJECT IDENTIFIER | 1.2.840.10045.4.1 | Other OIDs |
|  | parameters | NULL | NULL | NULL | Non-NULL |
| Signature Value | signatureValue | r | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | signatureValue | s | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | NOTE: |  |  |  |  |
|  | AP-Title (Airborne) | 1.3.27.1.Integer(1..(2**24-1)).Integer(0, 2).Integer(0..10),Integer(1-(2**8-1)) |  |  | See ICAO Doc 9705 Section 4.3 |
|  | AP-Title (Ground) | 1.3.27.2.Integer(1..?).Integer(0, 2).Integer(0..10),Integer(1-(2**8-1)) |  |  | See ICAO Doc 9705 Section 4.3 |
|  | NET (Fixed AINSC) | 47 0027 01 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Mobile AINSC) | 47 0027 41 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Fixed ATSC) | 47 0027 81 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Mobile ATSC) | 47 0027 C1 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | AMHS Address | X.400 Address |  |  |  |

This page intentionally left blank.

# PKI Laboratory Environment

### Entrust CA/RA

#### Description

Entrust PKI 5.1.1 (now known as Entrust Authority), produced by Entrust Technologies, is a combined Certificate Authority (CA) and Registration Authority (RA).  The product’s main purpose is for producing Digital Ids, to be used for system and application logins.  Entrust PKI Digital Ids can also be used for signing and encrypting documents.  Entrust PKI does not contain a standalone toolkit for creating customized applications.

#### Environment—Entrust CA

The Entrust CA is hosted on a Pentium 200 with 128MB of RAM, running Windows NT Server.  The version of Entrust CA currently installed in the PKI Lab is v5.1.1.  Entrust CA stores its data in an Informix database (supplied with the Entrust product), and publishes certificates and CRLs to a PeerLogic x.500 Directory (also supplied).

#### Environment—Entrust RA

The Entrust RA is hosted on a Pentium 200 with 64MB of RAM, running Windows NT Workstation.  The version of Entrust RA currently installed in the PKI Lab is v5.1.1.  Entrust RA is a client to Entrust CA.

The PeerLogic x.500 Directory Server was supplied with Entrust RA, and it is co-hosted on the Entrust RA system.

### Xcert Sentry CA

#### Description

SentryCA, a product formerly produced by Xcert International, Inc., was purchased by RSA Technologies in February 2001.  RSA is replacing it’s own Bsafe product with SentryCA, and SentryCA will adopt the Bsafe name.  Xcert also produced the Xcert Development Kit (XDK), which is a software toolkit distributed with the SentryCA product for developing stand-alone PKI applications or for integration with SentryCA.

#### Environment

The SentryCA server is hosted on a Pentium 200 with 128MB of RAM, running Windows 2000 Server.  SentryCA is web-based, and as such requires Microsoft Internet Information Server (IIS), which is only available on the Microsoft Server platforms.  The version of SentryCA currently installed in the PKI Lab is v4.5.  SentryCA has it’s own LDAP server, but is capable of publishing to any x.500 directory.

### UniCERT CA

#### Description

The UniCERT product, which is produced by Baltimore Technologies, consists of four components. UniCERT CA manages digital certificates.  UniCERT CAO manages the CA security policy.  UniCERT RA divides the entire product into “operational domains” – assigning certain responsibilities to each part.  UniCERT RAO approves certificate requests to be issued by the CA.  UniCERT is based on an Oracle database, although certificates can be exported to an LDAP database as well.

#### Environment

The UniCERT CA is a Pentium 200 with 128MB of RAM, running Windows 2000 Server.  The version of UniCERT CA currently installed in the PKI Lab is v3.5.1.  This version is the only version that is certified by Baltimore Technologies to run on the Windows 2000 platform, and requires version 8.1.6 (also known as 8i) of Oracle, which is also installed.

### Certicom Trustpoint

#### Description

Certicom Trustpoint is a highly configurable CA product.  Trustpoint is comprised of a CA, which generates certificates, and the PKI Portal, a front-end application for the certificate request process.  Both CA and Portal are written in JAVA, and can be configured by modifying various class files.  Trustpoint stores certificates in an Oracle database, but can be configured to publish to an LDAP database as well.

#### Environment

Trustpoint (both CA and PKI Portal) and required Oracle 8i database are hosted on a Pentium 466 with 256MB of RAM, running Windows 2000 Server.  The version of Trustpoint that is currently installed in the PKI Lab is v1.5.   Trustpoint is also configured to publish to an iPlanet Directory Server (v5.0), as recommended in the Trustpoint installation guide.

# Detailed Analysis and Defect Report

## Entrust PKI 5.1.1

In laboratory testing, Honeywell was not able to produce an ECC certificate using the Entrust product.  Entrust support engineers have indicated that the version used for testing (5.1.1) does not allow for CA ECDSA signing; the forthcoming version 6.0 will include this feature.  Entrust exports certificates in DER format, and will allow for the selection of custom curves in version 6.0.  SubjectAlternateName and IssuerAlternateName are supported in version 5.1.1, however SignatureAlgorithm will not be supported until version 6.0, along with KeyUsage.  The current version of Entrust CA inserts Entrust-specific fields into each certificate, and so is not capable of producing a compressed certificate at this time.

## RSA (Xcert) Sentry CA

While not meeting all ATN Security Provisions in Sub-Volume VII, Xcert SentryCA (now known as RSA Bsafe) meets many ATN requirements.

- ECC Support—The tested version of SentryCA supports ECC and includes two standard Certicom curves, 163-bit and 239-bit.  Using the XDK toolkit, SentryCA supports programming of other curves, such as those specified for ATN (note that this programming was not performed as part of this validation activity).  As tested, the SentryCA permitted the creation of a self-signed CA certificate that is signed using ECC.  However, SentryCA does not support generation of ECC subject public keys (only RSA); the vendor has indicated that it is possible develop a custom application for this capability using the XDK toolkit.

- Extensions—SentryCA supports all ATN-required x.509 extensions.

- Field Values—Some noted defects are attributable to errors in the draft standards, and they have been corrected in subsequent versions that are currently available.  For instance, a typographical error in the OID value for the “signature algorithm” and “algorithm OID” fields has been corrected in the latest version of SentryCA.  In all four ATN-specified extensions, the Critical field does not explicitly contain the Boolean value for “Non-Critical,” as required in the ATN SARPs.

- Encoding—The SentryCA supports DER, which is required by ATN for ground-based certificates.  SentryCA does not support PER encoding; however, this encoding is applied only to compressed certificates, which are not a standard certificate format of SentryCA.

- Compressed Certificates—The vendor has indicated that the XDK toolkit includes utilities that may be useful in creating compressed ATN certificates (note that this was not verified as part of this validation activity).

The detailed analysis is presented in Figure 3.

## Baltimore UniCERT

In laboratory testing, Honeywell was unable to configure the UniCERT product to support ECC. In addition, Baltimore support engineers indicated that this product does not include full support for standard x.509 extensions.

## CERTICOM Trustpoint

Through the use of configurable policies, Certicom Trustpoint is capable of producing an ATN certificate with valid data in all required fields, with the exception of explicitly marking extension fields as non-critical status.

- ECC Support — The tested version of Trustpoint supports ECC and 27 standard curves, including the ATN Curves (SECT163R2 and SECT233R1).  The CA produced an ECC self-signed certificate using a non-ATN ECC curve, and it generated ECC user certificates using the ATN-specified SECT163R2 curve.  Certicom has indicated that the ATN-specified curve for CA self-signed certificates is planned for a future release.

- Extensions — Trustpoint CA supports all ATN-required x.509 extensions through the use of customized Certificate Policies.

- Field Values—All values have been verified to be correct with the following exceptions:

- In all four ATN-specified extensions, the Critical field does not explicitly contain the Boolean value for “Non-Critical,” as required in the ATN SARPs.  (Note: During testing, when the field was marked as “Critical” rather than “Non-critical,” the field explicitly contained the “Critical” Boolean value.)

- The KeyUsage extension has been verified to be correct with the “Digital Signature” extension value; however, the policy used to generate the certificates was not tested for “Key Agreement” or “Certificate/CRL Sign” extension values.

- In the AuthorityKeyIdentifier extension, the KeyIdentifier value is DER encoded and further investigation is necessary to ascertain whether that value is indeed correct.

- Encoding—Trustpoint CA supports DER encoding, which is required by ATN for ground-based certificates.  This product does not support PER encoding; however, this encoding is applied only to compressed certificates, which are not a standard certificate format of Trustpoint.

- Compressed Certificates—The vendor has indicated that through customization of the product itself, Truspoint can be tailored to support compressed ATN certificates (note that this was not verified as part of this validation activity).

The detailed analysis is presented in Figure 4.

### Figure 3. Detailed Analysis of RSA (formerly Xcert) SentryCA Certificate

| ATN Certificate Structure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Title | ASN.1 Name | Valid Values | Invalid Values | SentryCA Values | Notes |
| version |  | 2 | 0, 1 | 2 | Meets Requirement |
| serialNumber | CertificateSerialNumber | Any | Non-Integer | 00c6ba38820000027c0000000500000008 | Meets Requirement |
| signature | Algorithm | 1.2.840.10045.4.1 | Other OIDs | 1.2.840.10045.1 (certicomAlgorithm-e cdsa-with-sha1) | Meets Requirement (Correct algorithm; typographical error in OID value has been corrected by vendor in subsequent version) |
|  | Parameters | NULL | Non-NULL | <NULL> | Meets Requirement |
| issuer | AttributeType | X.501 Distinguished Name Types | Non-X.501 Distinguished Name | No Value | Fails Requirements – No Value (requires further evaluation) |
|  | AttributeValue | Type Dependent | Type Dependent |  | Fails Requirements – No Value (requires further evaluation) |
| validity | NotBefore | YYMMDDhhmmssZ | Non-YYMMDDhhmmssZ | 010502204031Z | Meets Requirement |
|  | NotAfter | YYMMDDhhmmssZ | Non-YYMMDDhhmmssZ | 040428201855Z | Meets Requirement |
| subject | AttributeType | X.501 Distinguished Name Types / Empty | Empty / Non-X.501 Distinguished Name Types | 2.5.4.3 (id-at-commonName) (U)(P) Printable String(13) 9(9) ::= P Johnson}}} | Meets Requirement |
|  | AttributeValue | Type Dependent | Type Dependent | No Value | Meets Requirement |
| subjectPublicKeyInfo | Algorithm | 1.2.840.10045.2.1 |  | 1.2.840.113549.1.1.1 | Fails Requirements – Improper Algorithm (RSA rather than ECC) |
|  | Parameters | 1.3.132.0.15 1.3.132.0.27 | Other OIDs | <NULL> | Fails Requirements – Improper value |
|  | SubjectPublicKey | 02¦¦xQ (compressed) 03¦¦xQ (compressed) 04¦¦xQ¦¦yQ (uncompressed) | Any other form | 00a715fc1429cbfa18f3f681ca5694ccc6239311bc50964b0905e2a6151d11b25a3cc6181e3dc3c4912a2d0bb0f9e6d450241cef5d9518215bada93d80bb1e3d79 | Fails Requirements – Improper Value (RSA key rather than ECC) |
| issuerUniqueID |  | Omitted | Any |  | Meets Requirement |
| subjectUniqueID |  | Omitted | Any |  | Meets Requirement |
| Extensions |  |  |  |  |  |
| AuthorityKeyIdentifier | ExtnID | 2.5.29.35 | Other OIDs | 2.5.29.35 (id-ce-authorityKeyIdentifier) | Meets Requirement |
|  | Critical | FALSE | TRUE |  | Fails Requirements – Does not explicitly include this field for extensions marked non-critical |
|  | ExtnValue – keyIdentifier | 0100 plus 60 least significant bits of the SHA-1 hash of the subjectPublicKey | Other than Valid Values | 193dd18a959397fed61c227bd11d65d89ea192bb | Fails Requirements – Improper Value (based on RSA subjectPublicKey) |
|  | ExtnValue – authorityCertIssuer | Omitted | Any |  | Meets Requirement |
|  | ExtnValue - authorityCertSerialNumber | Omitted | Any |  | Meets Requirement |
| KeyUsage | extnID | 2.5.29.15 | Other OIDs | 2.5.29.15 (id-ce-keyUsage) | Meets Requirement |
|  | critical | FALSE | TRUE |  | Fails Requirements – Does not explicitly include this field for extensions marked non-critical |
|  | extnValue - Key Usage | 00000001 00010000 01100000 | All other combinations | 0106 | Requires further investigaion - may be encoded |
| SubjectAltName | extnID | 2.5.29.17 | Other OIDs | 2.5.29.17 (id-ce-subjectAltName) | Meets Requirement |
|  | critical | FALSE | TRUE |  | Fails Requirements – Does not explicitly include this field for extensions marked non-critical |
|  | extnValue - SubjectAltName | 3: AMHS Address 7:NET OCTET STRING SIZE (20) 8:AP-Title | 0: 1: 2: 4: 5: 6: | 1.3.19.78 (1.3.19.78) | Meets Requirement |
| IssuerAltName | extnID | 2.5.29.18 | Other OIDs | 2.5.29.18 (id-ce-issuerAltName) | Meets Requirement |
|  | critical | FALSE | TRUE |  | Fails Requirements – Does not explicitly include this field for extensions marked non-critical |
|  | extnValue - issuerAltName | 8:AP-Title (See Note) | 0: 1: 2: 3: 4: 5: 6: 7: | 2.2.19.76 (2.2.19.76) | Meets Requirement |
| Algorithm OID | algorithm | 1.2.840.10045.4.1 | Other OIDs | 1.2.840.10045.1 (certicomAlgorithm-ecdsa-with-sha1) | Meets Requirement (Correct algorithm; typographical error in OID value has been corrected by vendor in subsequent version) |
|  | parameters | NULL | Non-NULL | <NULL> | Meets Requirement |
| Signature Value | signatureValue | 1 to n-1 | Other than 1 to n-1 | 00302e02150188cf6a129770450d974ba4a61e6baef109c34d46021500ee7e93002f2001d5f11fad65dde63a8447f0f4ff | Meets Requirement |

### This page intentionally left blank.

### Figure 4. Detailed Analysis of Certicom Trustpoint Certificate

| ATN Certificate Structure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Title | ASN.1 Name | Valid Values | Invalid Values | Trustpoint Values | Notes |
| version |  | 2 | 0, 1 | 2 | Meets Requirement |
| serialNumber | CertificateSerialNumber | Any | Non-Integer | 402 | Meets Requirement |
| signature | Algorithm | 1.2.840.10045.4.1 | Other OIDs | 1.2.840.10045.4.1 | Meets Requirement |
|  | Parameters | NULL | Non-NULL | NULL | Meets Requirement |
| issuer | AttributeType | X.501 Distinguished Name Types | Non-X.501 Distinguished Name | DN of the CA | Meets Requirement |
|  | AttributeValue | Type Dependent | Type Dependent | Maryland, Columbia, Honeywell, ASTG, demo CA | Meets Requirement |
| validity | NotBefore | YYMMDDhhmmssZ | Non-YYMMDDhhmmssZ | 011220164149Z | Meets Requirement |
|  | NotAfter | YYMMDDhhmmssZ | Non-YYMMDDhhmmssZ | 020620154030Z | Meets Requirement |
| subject | AttributeType | X.501 Distinguished Name Types / Empty | Empty / Non-X.501 Distinguished Name Types | DN of the Subject | Meets Requirement |
|  | AttributeValue | Type Dependent | Type Dependent | demo Portal Admin | Meets Requirement |
| subjectPublicKeyInfo | Algorithm | 1.2.840.10045.2.1 |  | 1.2.840.10045.2.1 | Meets Requirement |
|  | Parameters | 1.3.132.0.15 1.3.132.0.27 | Other OIDs | 1.3.132.0.15 | Meets Requirement |
|  | SubjectPublicKey | 02¦¦xQ (compressed) 03¦¦xQ (compressed) 04¦¦xQ¦¦yQ (uncompressed) | Any other form | 03 02 52 43 36 86 9B D5 1D A0 21 AD B4 56 B5 6B 6C 9D 02 D4 E3 E | Meets Requirement |
| issuerUniqueID |  | Omitted | Any |  | Meets Requirement |
| subjectUniqueID |  | Omitted | Any |  | Meets Requirement |
| Extensions |  |  |  |  |  |
| AuthorityKeyIdentifier | ExtnID | 2.5.29.35 | Other OIDs | 2.5.29.35 | Meets Requirement |
|  | Critical | FALSE | TRUE |  | Fails Requirements – Does not explicitly include this field for extensions marked non-critical |
|  | ExtnValue – keyIdentifier | 0100 plus 60 least significant bits of the SHA-1 hash of the subjectPublicKey | Other than Valid Values | 30 16 80 14 63 8C 26 36 62 99 7F 31 AB AC 1B 40 BA 7C 40 7D D4 FA 63 6C | Meets Requirement |
|  | ExtnValue – authorityCertIssuer | Omitted | Any |  | Meets Requirement |
|  | ExtnValue - authorityCertSerialNumber | Omitted | Any |  | Meets Requirement |
| KeyUsage | extnID | 2.5.29.15 | Other OIDs | 2.5.29.15 | Meets Requirement |
|  | critical | FALSE | TRUE |  | Fails Requirements – Does not explicitly include this field for extensions marked non-critical |
|  | extnValue - Key Usage | 00000001 00010000 01100000 | All other combinations | 03 02 07 80 | Meets Requirement (Digital Signature) |
| SubjectAltName | extnID | 2.5.29.17 | Other OIDs | 2.5.29.17 | Meets Requirement |
|  | critical | FALSE | TRUE |  | Fails Requirements – Does not explicitly include this field for extensions marked non-critical |
|  | extnValue - SubjectAltName | 3: AMHS Address 7:NET OCTET STRING SIZE (20) 8:AP-Title | 0: 1: 2: 4: 5: 6: | 30 1D 81 1B 70 61 74 74 69 2E 6A 6F 68 6E 73 6F 6E 40 68 6F 6E 65 79 77 65 6C 6C 2E 63 6F 6D | Meets Requirement |
| IssuerAltName | extnID | 2.5.29.18 | Other OIDs | 2.5.29.18 | Meets Requirement |
|  | critical | FALSE | TRUE |  | Fails Requirements – Does not explicitly include this field for extensions marked non-critical |
|  | extnValue - issuerAltName | 8:AP-Title (See Note) | 0: 1: 2: 3: 4: 5: 6: 7: | 30 12 81 10 63 61 40 68 6F 6E 65 79 77 65 6C 6C 2E 63 6F 6D | Meets Requirement |
| Algorithm OID | algorithm | 1.2.840.10045.4.1 | Other OIDs | 1.2.840.10045.4.1 | Meets Requirement |
|  | parameters | NULL | Non-NULL | NULL | Meets Requirement |
| Signature Value | signatureValue | 1 to n-1 | Other than 1 to n-1 | 30 2E 02 15 02 FF 91 0C 79 F0 A6 CF 7D DA 05 05 77 64 8E 92 96 C9 BD A4 E4 02 15 00 CF F2 43 71 61 56 3D 2C 69 4F A8 E3 3E 51 94 3C F2 C0 FB EF | Meets Requirement |

# RESUlts and analysis

## FVO 6

To determine if provision has been made to ensure that Sub-Volume VIII are implementation independent.

As determined by PKI validation analysis and inspection completed to date, the ATN PKI Certificates requirements could be implemented almost completely via off-the-shelf PKI systems from multiple vendors.  In cases where the off-the-shelf solutions do not provide certain required functionality, some vendors have indicated that those features are on their current product development schedule.  In those cases where a vendor has not yet committed to implement a required function, the nature of the requirements is such that they are not technically challenging.  The reason that they are not on a vendor’s product development schedule is due primarily to the lack of broad market demand for the functionality.  The following table summarizes the requirements that are not implemented and vendor responses to implementation schedules.

| Requirement | Entrust | RSA (formerly Xcert) | Baltimore Technologies | Certicom |
| --- | --- | --- | --- | --- |
| Extension—Issuer Alternate Name | Extension supported (unverified) | Extension supported; however not explicitly marked as “non-critical” | Extension not Supported | Extension supported; however not explicitly marked as “non-critical” |
| Extension—Subject Alternate Name | Modifiable to permit any documented or undocumented syntax by using the E-mail (subjectAltName)field | Extension supported; however not explicitly marked as “non-critical” | Extension supported (unverified) | Extension supported; however not explicitly marked as “non-critical” |
| Extension— Key Usage | Extension supported (unverified) | Extension supported; however not explicitly marked as “non-critical” | Extension supported (unverified) | Extension supported; however not explicitly marked as “non-critical” |
| User-Specified Elliptic Curve Parameters | Configuration of EC parameters is possible by providing detailed parameter information; however, this is not part of Entrust’s standard product validation. | Configuration of EC parameters is possible using the Xcert Development Kit (XDK). This has not been verified as part of this validation activity. | Not available, not currently scheduled, but being considered | Supported, including ATN curves |
| Compressed Certificates | Current version inserts Entrust-specific fields into each certificate, so it is not capable of producing a compressed certificate at this time. | Generation of compressed certificates may be possible using the Xcert Development Kit (XDK). This has not been verified as part of this validation activity. | Not supported. | Generation of compressed certificates would be possible via product customization by Certicom. This has not been verified as part of this validation activity. |
| Encoding | Not supported. | DER supported. | Not supported. | DER supported |

## TVO 8

To determine if the functionality described in Sub-Volume VIII is implementable.

As determined by PKI validation analysis and inspection completed to date, the ATN PKI Certificates requirements could be implemented almost completely via off-the-shelf PKI systems.  In cases where the off-the-shelf solutions do not provide certain required functionality, some vendors have indicated that those features are on their current product development schedule.  In those cases where a vendor has not yet committed to implement a required function, the nature of the requirements is such that they are not technically challenging.  The reason that they are not on a vendor’s product development schedule is due primarily to the lack of broad market demand for the functionality.

# Conclusions

Of all the PKI products tested to date, the off-the-shelf Certicom Trustpoint CA is almost fully compliant with all of the ATN PKI requirements.  In addition, with vendor customization, a tailored version of the product should be capable of producing ATN compressed certificates as well as the standard ATN certificates that it currently generates.  Trustpoint is the only product evaluated that supported the ATN ECC curves.

The off-the-shelf RSA (formerly Xcert) SentryCA is reasonably close to meeting all of the ATN PKI requirements.  In addition, the product includes an extensive PKI toolkit (XDK) that permits application customization, including specification of ATN-specific elliptic curves and potential generation of ATN compressed certificates.

Entrust has indicated that future versions of the Entrust CA/RA product are scheduled to include features that meet ATN requirements; however, those versions were not available for the PKI validation effort completed in calendar year 2001.

Baltimore Technologies’ UniCERT product does not currently include full support for standard X.509 extensions, and it includes only limited ECC support.