WGN01-WP12

AERONATICAL MOBILE COMMUNICATIONS PANEL (AMCP)

Working Group N - Networking

Working Paper

Establishing Connectivity Awareness Between ATN End Systems and IPv6 Subnetworks

Prepared by: Leon Sayadian

Presented by: Kelly Kitchens (USA)

May 2003

SUMMARY This working paper responds to Action 5/3 of the ATNP SG B1 meeting of Sept 29 – Oct 10, 2002 in Toulouse, France, which requested an investigation of IPv6 for its applicability in establishing ‘path liveness’ (e.g., emulating ESH and ISH with ICMPv6 messaging).

# 1. Scope

This paper provides guidance for implementing address mapping for ATN sub-networks that use the Internet Protocol version 6 (IPv6), and discusses how the “hello” messages are emulated between End Systems (ESs) and Intermediate Systems (ISs) in an IPv6 infrastructure.

# 2. Background

The ATN, as specified in reference [1], is required to implement the ES-IS protocol, which allows ESs and ISs to discover each other in accordance with ISO 9542 in the context of CLNP.

ES-IS was initially specified as the means for End Systems and Intermediate Systems to discover each other using the X.25 protocol [1]. Since X.25 services are rapidly vanishing from vendor offerings, an initiative is underway within ICAO ATNP to encapsulate ATN traffic over the commercially available IP network infrastructure. With the anticipation of migration to IPv6, Subgroup B has approved the following working papers to address the impact on ATN:

- Encapsulating ATN CLNP over IPv6 [2]

- IP SNDCF [3]

- Mapping ATN to IPv6 addresses [4, 8]

As a result of these efforts, an issue was raised regarding the establishment of a mechanism to mutually apprise IPv6 nodes of ‘common path liveness’ (i.e., acknowledgement of connectivity between nodes over their common link), analogous to ES Hello (ESH) and IS Hello (ISH) messaging in the canonical ATN documentation. This paper will discuss a candidate approach to implement such a mechanism within an IPv6 domain.

# 3. References

| 1 | ICAO Doc 9705, 3rd Edition: 2002 | Manual of Technical Provisions for the Aeronautical Telecommunications Network (ATN) |
| --- | --- | --- |
| 2 | ATNP/WGB-1/WP-305, 3/02 | Encapsulating ATN CLNP over an IPv6 Infrastructure |
| 3 | IP SNDCF: 2002 | Proposed Guidance Material In support of the IP SNDCF |
| 4 | ATNP/WGB-1/WP-304, 3/02 | Mapping Between ATN NSAP and IPv6 Addresses |
| 5 | IETF RFC 2460: 1998 | Internet Protocol, Version 6 (IPv6) Specification |
| 6 | IETF RFC 2463: 1998 | ICMPv6 for the IPv6 Specification |
| 7 | IETF RFC 2461: 1998 | Neighbor Discovery for IPv6 |
| 8 | ATNP/WGB-1/WP 405, 8/02 | Additional Mapping Formats Between Network Access Point Addresses (NSAPA) and IPv6 Addresses |

# 4. Assumptions

- ATN End Systems interface over IPv6/ICMPv6 sub-networks, as per [5, 6]

- CLNP is encapsulated over IPv6, as per [2]

- IP SNDCF is enabled, as per [3]

# 5. Discussion

Awareness of connectivity between hosts and routers is maintained in the ATN with the use of the ES-IS capability in the CLNP [1]. These connections may be point-to-point WANs, LANs, or general X.25 sub-networks. To support the transition to IPv6 sub-networks within the ATN, similar connectivity awareness is required between End Systems and IPv6 routers.

Within the ES-IS protocol, two messages are used to convey awareness of adjacency:

- ES Hello (ESH) messages are issued by ESs to announce themselves to ISs, and convey network and data-link layer addresses of the source ES

- IS Hello (ISH) messages are issued by ISs to announce themselves to ESs, and convey network and data-link layer addresses of the source IS

Figure 5-1 depicts the formats of the ESH and ISH packets.

Figure 5-1. ESH and ISH Packet Formats

In the IPv6 domain, similar functionality is available in the Neighbor Discovery protocol [7]. This protocol leverages the latent ICMPv6 [6] to send messages between hosts, routers, and other neighboring network nodes to mutually alert their connectivity in the network infrastructure.

This protocol includes a set of messages and processes that determine relationships between neighboring nodes. Particular applications include:

- Host discovery of adjacent routers

- Host discovery of addresses, address prefixes, and other configuration parameters

- Router discovery of adjacent hosts, and conveyance of configuration parameters and on-link prefixes

Additional options beyond the scope of this paper include mutual neighbor discovery, and redirect of messaging.

The objective of this paper is to describe a ‘path-liveness’ capability between ATN End Systems and adjacent IPv6 routers. This is accomplished within the IP SNDCF by mapping between comparable ES-IS and IPv6 messages. Table 5-1 describes periodic IPv6 messages of interest and their analogous ES-IS functions.

| Connectivity Awareness Function | IPv6 Neighbor Discovery Protocol Message | ##### ATN CLNP/ES-IS Message |
| --- | --- | --- |
| Host announcement to router(s) | Router Solicitation | ESH |
| Router announcement to host | Router Advertisement | ISH |

Table 5-1. Comparison of connectivity awareness capabilities between IPv6 and ISO

Formats of the ICMPv6 portions of the Router Solicitation and Router Advertisement messages are shown in Figure 5-2.

Type

Code

Checksum

Reserved

Options

Solicitation (Host-to-Router)

Field Length (byte)

4

2

1

1

Variable

Advertisement (Router-to-Host)

1

2

4

4

1

2

1

1

Variable

Type

Field Length (byte)

Router

Life Time

Reachable Time

Retransmit Timer

Options

Checksum

Cur Hop Limit

Reserved

Code

###### Figure 5-2. ICMPv6 Packet Formats for Router Solicitation and Advertisement

In order to emulate ESH messages within the IPv6 sub-network, analogous Router Solicitation messages are created in the IP SNDCF, where relevant fields from the ESH are mapped to parameter values of the Router Solicitation. Table 5-2 shows this mapping, where italicized Router Solicitation Parameters are in the IPv6 header portion of the packet, and the remaining parameters are in the ICMPv6 portion.

| ATN ESH Fields | Router Solicitation Parameters | Comments |
| --- | --- | --- |
| Source Address | Source Address | Map source NSAPA to IPv6 address as per [4, 8] |
| N/A | Destination Address | Use all-routers IPv6 multicast address |
| N/A | Hop Limit | Use default value of 255 |
| N/A | Authentication Header | Option if authentication security is to be invoked |
| N/A | Type | Use default value of 133 |
| N/A | Code | Use default value of 0 |
| N/A | Checksum | ICMPv6 calculates the checksum |
| N/A | Reserved | -- |
| (see comment) | Source Link-Layer Address | Optional for conveying LAN MAC address of ES |

Table 5-2. Mapping of ATN ESH to IPv6 Router Solicitation

In order to emulate Router Advertisements to the End System, analogous ATN ISH messages are created in the IP SNDCF, where relevant fields from the Router Advertisement are mapped to parameter values of the ATN ISH. Table 5-3 shows this mapping, which also includes an Options field for the ATN ISH.

| Router Advertisement Parameters | ATN ISH Fields | Comments |
| --- | --- | --- |
| N/A | Network Layer Protocol Identifier | Use default 1000 0010 |
| N/A | Length Indicator | Calculated by the IP SNDCF |
| N/A | Version/Protocol ID Extension | Use default 0000 0001 |
| N/A | Reserved | -- |
| N/A | Type | Use 00100 |
| Router Lifetime | Holding Time | Similar parameters |
| N/A | Checksum | Set to 0 |
| N/A | Network Entity Title Length Indicator | IP SNDCF calculates this parameter based on the NET |
| Source Address | Network Entity Title (NET) | Embed IPv6 Source Address in the NET as per RFC 1888, Section 6 |
| (See comment) | Options | Certain Router Advertisement parameters (e.g., Authentication Header) may be mapped to ISO 8473 optional parameters (i.e., Security, Priority, Suggested ES Configuration Timer) in this field |

Table 5-3. Mapping of IPv6 Router Advertisement to ATN ISH

# 6.   Recommendation

In the context of prior ATN SG B1 analyses of particular issues for interfacing ATN end systems with IPv6 sub-networks, this paper recommends the use of the IPv6-inherent Neighbor Discovery Protocol for informing contiguous hosts and routers of their mutual connectivity. In particular, Router Solicitations and Advertisements may be mapped to ATN ESH and ISH messaging in the IP SNDCF in crossing between the IPv6 and ATN domains.

# 7.   Abbreviation

ATM	Air Traffic Management

ATN	Aeronautical Telecommunication Network

CLNP	Connection-Less Network Protocol

ES	End System

ESH	End System Hello

ICAO	International Civil Aviation Organization

ICMP	Internet Control Message Protocol

IETF	Internet Engineering Task Force

IP	Internet Protocol

ISO	International Standards for Organization

IS	Intermediate System

ISH	Intermediate System Hello

LAN	Local Area Network

NSAPA	Network Service Access Point Addresses

OSI	Open System Interconnection

RFC	Request for Comment

SNDCF  Sub-Network Dependent Convergence Function

WAN	Wide Area Network